| Function | Pin Name | Pin Number | Connect to | Description |
| --- | --- | --- | --- | --- |
| Voltage Input | DVCC | 1 | 3.3V Regulator Output | Digital Supply Voltage |
| GND | DVSS | 20 | Ground | Ground Reference |
| SCL line | UCB0SCL | 14 | LCD | USCI_B0 I2C mode: SCL I2C clock |
| SDA line | UCB0SDA | 15 | LCD | USCI_B0 I2C mode: SDA I2C data |
| Echo line | P2.0 | 8 | Ultrasonic Sensor, MSP with voltage divider | General-purpose digital I/O pin |
| Trigger Line | P2.1 | 9 | Ultrasonic Sensor, MSP | General-purpose digital I/O pin |
| ADC input line | A3 | 5 | Potentiometer | ADC10 analog input A3 |
| ADC output line | P1.4 | 6 | LED | General-purpose digital I/O pin |
| MOSFET line | P1.5 | 7 | 1k Resistor, LED, 3.3V supply. | General-purpose digital I/O pin |
| Programmer Test | TEST | 17 | UART line, GND, dev board | Spy-Bi-Wire test clock input during programming and test |
| Programmer Reset | RST | 16 | UART line, pull-up 10k Ohm resistor, dev board | Spy-Bi-Wire test data input/output during programming and test |

3.1

A. Battery Pack

a. Terminal 1: Switch

b. Terminal 2: GND

B. Switch

a. Pin 1: Battery

b. Pin 2: NC

c. Pin 3: +3.3V & 5V

C. 3.3V regulator PCB pinouts (as female pin headers)

a. Pin 1: Vin

b. Pin 2: GND

c. Pin 3: GND

d. Pin 4: +3.3V

D. 5V regulator PCB pinouts (as female pin headers)

a. Pin 1: Vin

b. Pin 2: GND

c. Pin 3: GND

d. Pin 4: +5V

E. LCD display (as male pin headers)

a. Pin 1: GND

b. Pin 2: VCC (5V)

c. Pin 3: SDA (through pull up resistor)

d. Pin 4: SCL (through pull up resistor)

F. Ultrasonic Sensor (as female pin headers)

a. Pin 1: 5V Supply

b. Pin 2: Trigger Pulse Input

c. Pin 3: Echo Pulse Output

d. Pin 4: 0V Ground

G. Potentiometer

a. Pin 1: 3.3v

b. Pin 2: MSP ADC Port

c. Pin 3: Ground

H. BS170 MOSFET

a. Pin 1: Drain (3.3V supply through LED with 1k Ohm resistor)

b. Pin 2: Source (GND)

c. Pin 3: Gate (1k Ohm resistor, GPIO P1.5)

I. Programmer Pins

a. Pin 1: GND

b. Pin 2: spy-by-wire (Test Line) (TEST pin)

c. Pin 3: spy-by-wire (Reset Line) (RST pin) with pull-up 10k ohm resistor

d. Pin 4: NC

J. Extra IO Pins (Male header)

a. Pin 1: Port 1 - 1.0

b. Pin 2: Port 1 - 1.1

c. Pin 3: Port 1 - 1.2

d. Pin 4: +3.3v

e. Pin 5: Ground

f. Pin 6: +5.0v

Modified Code:
#define LCD_SCL            BIT6        // Pin 14 P1.6

#define LCD_SDA            BIT7        // Pin 15 P1.7

#define USENSOR_TRIGGER    BIT1        // Pin 9 P2.1

#define ECHO_INTERUPT      BIT0        // Pin 8 P2.0

#define ADC_INPUTPIN       BIT3        // Pin 5 P1.3

#define ADC_LED            BIT4        // Pin 6 P1.4

#define ADC_INPUT          INCH_3      // Channel A3

Schematic for Project Board: